PREFACE:

This manual is provided as a technical reference for DOS based

development using Turtle Beach Systems MultiSound Multi-Media audio

card.  It gives the user the capability to access all the features of

the MultiSound through the DOS environment.  It provides the assembled

code for the DSP, and fully explains how to interact with that code.

It provides descriptions of all the registers accessible from the HOST,

and how to use them.  This is accomplished by first going over the

basics of how the card operates.  Then the reader is walked through

examples to show how the HOST is used to control the various features

of the card.  Finally this manual provides a reference section in the

back for easy access to the information contained in this manual.

This developers tool kit contains the following items:

1. 3-1/2" Floppy

This contains the source for the examples and the DSP

code for the 56001

??? Do a file list and short description

2. This Manual

INTRODUCTION:

This Introduction provides useful information which you should review

before attempting to use this manual.

This introduction covers the following topics:

+ What you should know before starting

+ What you need to have

+ The information this manual will provide you with

+ Some notes on using the sample applications

+ Conventions used by this manual

What you need to know

To start using this tool kit you will need have and understanding of

the following things:

+ Advanced knowledge of the C programming language

This manual assumes that the reader is experienced with

programming in C.  This tool kit uses extensive port writing,

IRQ handling, structures, and pointers.  If you are not

confident in your C we recommend that you review it before

starting, so you can better understand what is going on.

+ Knowledge of digital audio and MIDI

This manual assumes a certain amount of knowledge on these

topics.  Although the discussion will not get too in depth, we

make no attempt to explain MIDI and digital concepts in this

manual.  If you find yourself wanting to know more about a

topic or just plain lost, check out the FURTHER READING section

in the reference part of this manual.

What you need to have

To effectively use this tool kit to create applications for the

MultiSound card you will need the following items:

+ An IBM-PC Compatible

MultiSound is designed for the ISA bus standard on IBM-PC

compatibles.  You will need one of these to use it.  You will

be able to use the card on a 286 or better, however a 386 or

better is recommended for dealing with digital audio.

+ The MultiSound Audio Card

We assume you have one if you bought this kit.

+ A C Compiler

All the sample source code was written to be compiled using

Microsoft C 6.00A.  Although we recommend using this compiler,

any adept C compiler will work with modest modifications to the

code.

+ A line level audio source

To go through the tutorial and test the code, you will need a

line level audio source( tape or CD player, mixer output,

etc..).  This will allow you to have something to check the

recording with, as well as check the AUX input.

+ A MIDI Keyboard

To fully explore the capabilities of the MultiSound, it is

suggested that the user have an external MIDI keyboard.

What is contained in this manual

This manual is broken up into five sections.

+ Introduction

The introduction provides an overview of the tool kit.  It

tells the user what he will need to use the tool kit, as well

as what knowledge he can expect to gain from it.

+ CHAPTER 1 - System Overview

This chapter goes over the hardware and software basics

necessary to understand programming the multisound

card.  It also explains the data formats used in the demo code,

to make this manual more readable.

+ CHAPTER 2 - Initializing the Hardware

This chapter covers how to initialize the MultiSound.  This

will be used in every application you write for the card.

+ CHAPTER 3 - Queues From Hell

This chapter gives the reader an overview of the queue based

software architecture used for programming the MultiSound.

+ CHAPTER 4 - Using Digital Audio

This chapter covers digital audio on the MultiSound.  It takes

the reader through examples for both playing and recording.

+ CHAPTER 5 - Using MIDI

+ CHAPTER 6 - The Reference

This section is provided as a quick reference for readers who

have all ready read this manual.

Whats up with those sample applications

All the source for the tutorial examples are contained on the

included disk.  The file MCONTROL.C is linked with every sample

application.  This file contains a series of functions and

structures which are used to access the MultiSound.  It should be

remembered that these source files show only one system for

interfacing with the card.  As with any programming task there is

no absolute right.  The important thing to understand when going

through this manual is the low level interaction between the DSP

and the HOST( IBM-PC compatible platform ).

Conventions used by this manual

Fill this area in later

CHAPTER 1 - A System Overview

Introduction

Hardware Overview

+ I/O Port Usage

+ Memory Addressing PC + DSP

+ Memory Map ( SMA )

Software Overview

+ Structs/Unions, typedefs, and MACROS used

Introduction

This chapter deals with the ground floor of development for the

MultiSound.  It will take you through the hardware, explaining how you

will communicate with the card.  Then it goes over the registers

accessible from the PC.  Finally it takes you through the steps of

initializing the card, and ends with a short application for adjusting

the input volumes.

Hardware Overview

MultiSound is a full length, 16 bit, AT compatible feature card that

allows real-time digital signal processing to be performed on a PC.

The DSP used by the MultiSound is the Motorola DSP 56001, running with

a 40 Mhz clock. In this speed, the DSP performs 20 million instructions

per second.  The onboard synthesizer is a Proteus 1/XR which has 4

megabytes of sampled ROM, and has 384 preset sounds available.

Externally the card has two analog inputs, one analog output, a MIDI

port, and a game port.

The MultiSound interfaces to the host PC system via three hardware

resources:  I/O port address block, Interrupt, and 32Kb window of

shared memory.  The location and usage of these resources is

covered in the following paragraphs.

I/O Port Usage

This section covers the I/O port usage.  It gives the names and

locations of the registers.  Note that the names given to the registers

are defined in MSND_DSP.H using the same syntax.

The I/O port address options and settings are covered in the MultiSound

User's Guide under 'Installing the Hardware'.  There are eight possible

port starting locations, each occupying sixteen consecutive addresses.

In this document, the I/O base address will be referred to as

"xx0", so HP_CVR would be referred to at xx1h, regardless of the

base port selected.

The first eight I/O addresses are simply the host port interface of the

Motorola DSP56001, the remaining addresses are additional control

ports.  The DSP host port interface is used to control the flow of

and send data and exceptions between the DSP and the PC.  The remaining

addresses are the board information registers, and the AT bus interface

control registers.

Equates for I/O Ports

PU = Power-up State,  R/W = Read / Write,  RO = Read Only,  WO = Write Only

DSP Host Port Interface Registers:

HP_ICR		xx0H		Interrupt Control Register	(R/W)

HP_CVR		xx1H		Command Vector Register		(R/W)

HP_ISR		xx2H		Interrupt Status Register	(RO)

HP_IVR		xx3H		Interrupt Vector Register	(R/W)

HP_RXZ		xx4H		(a byte of zeros)			(RO)

HP_RXH		xx5H		Rx Data High				(RO)

HP_RXM		xx6H		Rx Data Middle				(RO)

HP_RXL		xx7H		Rx Data Low					(RO)

HP_TXN		xx4H		(not used)					(RO)

HP_TXH		xx5H		Tx Data High				(RO)

HP_TXM		xx6H		Tx Data Middle				(RO)

HP_TXL		xx7H		Tx Data Low					(RO)

For more information on the use of these registers consult the

DSP56000/DSP56001 Digital Signal Processor User's Manual from Motorola.

Chapter 10 of the Motorola manual is a gold mine of information about

the 56001 host port.

AT Bus Interface Control Registers (Write Only):

HP_MEMM		xx8H		Shared Memory Map Register

HP_IRQM		xx9H		IRQ # Mapping Register

HP_DSPR		xxAH		DSP Reset Control Register

HP_PROR		xxBH		Proteus Reset Control Register

HP_BLKS		xxCH		Memory Block Select Register

HP_WAIT		xxDH		Extra Wait State Register

HP_BITM		xxEH		8/16-bit Memory Mode Register

HP_RESERVED	xxFH		(future use)

Board Information Registers (Read Only):

HPR_BLRC	xx8H		Board Level R/C Timer Input

HPR_SPR1	xx9H		(future)

HPR_SPR2	xxAH		(future)

HPR_TCL0	xxBh		TOPCAT Chip Level - Bit 0

HPR_TCL1	xxCh		  "     "     "   - Bit 1

HPR_TCL2	xxDh		  "     "     "   - Bit 2

HPR_TCL3	xxEh		  "     "     "   - Bit 3

HPR_TCL4	xxFh		  "     "     "   - Bit 4

The following section will cover the use of the additional

registers.

HP_MEMM			xx8H			Shared Memory Map Register

This port controls the segment address of the shared RAM window in the PC's

address space.  The port is actually only 3 bits wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  Memory Map Sel |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_MEMM:

HPMEM_NONE	0	Off / Not Mapped			(PU)

HPMEM_B000	1	B000H-B7FFH

HPMEM_C800	2	C800H-CFFFH

HPMEM_B000	3	D000H-D7FFH

HPMEM_NU	4	not used

HPMEM_B000	5	D800H-DFFFH

HPMEM_B000	6	E000H-E7FFH

HPMEM_B000	7	E800H-EFFFH

HP_IRQM			xx9H			IRQ # Mapping Register

This port controls which PC interrupt is used for the DSP host port.

The port is actually only 3 bits wide.  NOTE: Since the polarity of

the host port and the PC interrupts are inverted, this map should only

be written during an active host port interrupt.  This is explained in

the section 'Initializing the Hardware'.

To do this, have the

PC's 8259 interrupt controller disabled (or CLI in PC assembler), then

enable transmit interrupts on the host port. This will set the

interrupt logic active (high), provided that the transmitter is empty.

Write the desired HP_IRQM value, then disable the transmit interrupt on

the host port.  Finally, re-enable the interrupt on the PC's 8259 (or

STI in PC assembler).

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |   IRQ Map Sel   |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_IRQM:

HPIRQ_NONE	0	None / Not Mapped			(PU)

HPIRQ_5		1	IRQ #5

HPIRQ_7		2	IRQ #7

HPIRQ_9		3	IRQ #9 (2)

HPIRQ_10	4	IRQ #10

HPIRQ_11	5	IRQ #11

HPIRQ_12	6	IRQ #12

HPIRQ_15	7	IRQ #15

HP_DSPR			xxAH			DSP Reset Control Register

This port allows the DSP to be put into or taken out of RESET mode.

The port is actually only 1 bit wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  0  |  0  | Rst |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_DSPR:

HPDSPRESET_OFF	0	DSP is Running

HPDSPRESET_ON	1	DSP is in Reset				(PU)

After a reset of the host computer, the host registers will be

inactive.  This is because the DSP is in reset mode.  Writing a 0 to

HP_DSPR allows the DSP out of reset mode.  Writing a 1 to HP_DSPR puts

the DSP back into reset.  The reset on to reset off cycle should have a

minimum time delay of 650 nsec.  The DSP is not very functional

immediately after reset since it is waiting for program code to be

uploaded through the host port.

HP_PROR			xxBH			Proteus Reset Control Register

This port allows the Proteus subsystem to be put into or taken out of

RESET mode.  The port is actually only 1 bit wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  0  |  0  | Rst |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_PROR:

HPPRORESET_OFF	0	Proteus is Running

HPPRORESET_ON	1	Proteus is in Reset			(PU)

After a reset of the host computer, the Proteus subsystem will be

inactive.  This is because the Proteus is in reset mode.  Writing a 0

to HP_PROR allows the Proteus out of reset mode.  Writing a 1 to

HP_PROR puts the Proteus back into reset.  The reset on to reset off

cycle should have a minimum time delay of 1 usec.  The Proteus is not

fully functional immediately after reset since it is waiting for a

complete set of presets to be uploaded (via MIDI sysex blocks).

HP_BLKS			xxCH			Memory Block Select Register

This port allows the PC to choose which of the two 32Kb blocks of DSP

RAM is currently viewed in the shared RAM window.  The PC window

segment is mapped based on HP_MEMM (as described above) and the RAM is

addressed from offset 0000H-7FFFH.  The port is actually only 1 bit

wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  0  |  0  | Blk |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HB_BLKS and Viewed DSP addresses:

HPBLKSEL_0	0	View 1st Block

X/P:4000H-7FFFH, Y:8000H-BFFFH		(PU)

HPBLKSEL_1	1	View 2nd Block

X/P:8000H-BFFFH, Y:4000H-7FFFH

HP_WAIT			xxDH			Extra Wait State Register

This port allows one additional wait state to be added to PC memory

accesses.  The port is actually only 1 bit wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  0  |  0  | 1WS |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_WAIT:

HPWAITSTATE_0	0	No Added Wait States			(PU)

HPWAITSTATE_1	1	One Additional Wait State

On some clones, because of non-compliance with the ISA Bus

specification, it is necessary to add one wait state to achieve proper

host memory operation.

HP_BITM			xxEH			8/16-bit Memory Mode Register

This port allows the selection of either 8 or 16 bit PC memory

accesses.  The port is actually only 1 bit wide.

7      6     5     4     3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+

|  0  |  0  |  0  |  0  |  0  |  0  |  0  | 8BM |

+-----+-----+-----+-----+-----+-----+-----+-----+

Values for HP_BITM:

HPBITMODE_16	0	16-bit mode (full bandwidth)		(PU)

HPBITMODE_8		1	8-bit mode (approx. 25% reduction)

On some clones, because of non-compliance with the ISA Bus

specification, it is necessary to run the bus in 8-bit mode, to achieve

proper host memory operation.

Memory Addressing

This section goes over the addressing scheme for both the DSP and the

PC.

MultiSound has 32K x 24 bits of memory. The DSP views this memory as

one continuous block.  Since the DSP data bus is 24 bits wide and the

PC data bus is 16 bits wide, the least significant 8 bits are not

available from the PC bus side. The PC memory is addressed in 8 bit

increments so 16K of DSP RAM appears as 32K of RAM on the PC side. This

is why the memory block selector (see above) is needed. It also

explains why the PC's window appears to be twice the size of the

related DSP RAM. While all types of RAM accesses will work from the PC

side, the most useful are word accesses since the DSP has no choice but

to view the RAM in words.  All translation of the data is taken care of

in the hardware.

Note that this RAM is truly shared. If the PC is using the memory, the

DSP will be put on hold during external bus cycles.

This shared memory area( SMA ) is used to buffer MIDI and audio data

going both in and out, as well as holding the queuing structures.  The

memory map of the SMA is shown below.  The usage of the individual

locations will be explained as we proceed.  For now just take a look

at it, and note what kind of data is held there.

.

.

.

??MEM MAP here

.

.

.

Software overview

The demo code uses several typedefs, macros, structures, and unions.

All of there definitions are contained in the file MCONTROL.H.  Here we

will briefly describe their utility.

If you have done any windows programming these typedefs will be

familiar to you

typedef unsigned char BYTE;

typedef unsigned int  WORD;

typedef unsigned long DWORD;

The following macros are provided to make the code more readable.

HIWORD(l)         These macros are used to extract WORDs from DWORDs

LOWORD(l)         Example: HIWORD ( 0x0000:FFFF ) = 0x0000

LOWORD ( 0x0000:FFFF ) = 0xFFFF

HIBYTE(l)         These macros are used to extract BYTEs from WORDs

LOBYTE(l)         Example: HIBYTE ( 0x00FF ) = 0x00

LOBYTE ( 0x00FF ) = 0xFF

MAKELONG(low,hi)  These macros are used to make a long from two WORDs

MAKEWORD(low,hi)  or a WORD from two BYTEs.

Example: MAKELONG( 0xFFFF , 0x0000 ) = 0x0000:FFFF

MAKEWORD( 0xFF , 0x00 )     = 0x00FF

PCTODSP_OFFSET(w) These macros are used to convert a PC offset address

PCTODSP_BASED(w)  into an offset or a based address readable by the

DSP.  Example: PCTODSP_OFFSET( 0x2400 ) = 0x1200

PCTODSP_BASED( 0x2400 ) = 0x1200 + 0x4000

CLI 			  These macros are just there assembly equivalent, and

STI				  are used to disable and re-enable interrupts.  They

are defined in MCONTROL.H as:

_asm { cli } and _asm { sti } respectively

?? give a description of the structures and exceptions here

For a full description of the structures and unions used see the

reference section of this manual.

CHAPTER 2 - Initializing the Hardware

This chapter will take you through the steps necessary to initialize

the hardware.  Then it will step you through the demo code AUXVOL.C and

INVOL.C which will allow you to adjust the input volumes of the

MultiSound.

There are four basic steps necessary to initializing the hardware.

They are as follows:

+ Reset DSP

+ Reset Proteus

+ Loading DSP Code

+ Initializing SMA

+ Setting up the IRQ

Each of these will be described in full in the following paragraphs.

Reset the DSP

The first step to initializing the card is resetting the DSP.  This is

accomplished using the HP_DSPR register as described in the I/O port

usage section of this manual.

int resetdsp()

{

int nTimeOutCount = 20000;

outp( wBASEIO + HP_DSPR , HPDSPRESET_ON );

// It takes approximately 650ns for the DSP to reset.

outp( wBASEIO + HP_DSPR , HPDSPRESET_OFF );

while(nTimeOutCount-- >= 0) {

if( inp(wBASEIO + HP_CVR) == 0x12 ) {

return(1);

}

}

return(0);

}

In this example the variable wBASEIO is equal to the I/O base port

address, and HP_DSPR is the offset of the host port DSP reset

register.  This function simply writes to the HP_DSPR register to make

the DSP go into reset.  Then it writes to the register again to take it

out of reset.  Then it polls the HP_CVR register to check for success.

If the variable nTimeOutCount decrements to zero, we return an error.

The DSP needs to be in reset for approximately 650ns.  In most cases

you will not have to worry about adding code for this delay.

Reset the Proteus

This is basically the same as the DSP reset, only we are writing to the

HP_PROR register instead of the HP_DSPR register.  Below is the

function in MCONTROL.C which accomplishes this task.

void ResetProteus( void )

{

int nCounter;

outp( wBASEIO + HP_PROR , HPPRORESET_ON );

for( nCounter = 0 ; nCounter > 100 ; nCounter++ );

// Proteus needs to be in reset for approx 1 usec

outp( wBASEIO + HP_PROR , HPPRORESET_OFF );

}

The only difference in this case is that some delay must be implemented

to insure that the Proteus remains in reset for approximately 1

microsecond.

Loading the DSP code

The specifics of uploading the DSP code are not relevant to the

material in this manual.  We provide you with two functions in

MCONTROL.C which allow you to do this.  Executing these two functions

in the shown order will upload the code to the DSP.

uploadreb ("msndperm.reb");

uploadbin ("msndinit.bin");

Initializing the Shared Memory Area( SMA )

Once the DSP code is loaded we have to set the SMA variables to their

default values.  The function initSMA() in MCONTROL.C is used by the

demo code to do this.  This function defaults the variables, as well as

setting the SMA offsets to the appropriate pointer values based on the

selected shared RAM base address.  You may want to modify the values

set in initSMA depending on your specific application.

Setting up the IRQ

The functions Setup56kIRQ() and SetIntMap( BYTE bIRQ ) are provided

to set up the Interrupt on the MultiSound.  For a further explanation

of PC-Interrupts consult one of the references in the FURTHER READING

section of this manual.

The SetIntMap function sets up the interrupt value received from the

MultiSound.  Since the polarity of the host port and the PC interrupts

are inverted, this map should only be written during an active host

port interrupt.  To do this, have the PC's 8259 interrupt controller

disabled (or CLI in PC assembler), then enable transmit interrupts on

the host port. This will set the interrupt logic active (high),

provided that the transmitter is empty ( waitTXDE() ).  Write the

desired HP_IRQM value, then disable the transmit interrupt on the host

port.  Finally, re-enable the interrupt on the PC's 8259 (or STI in PC

assembler).  The function waitTXDE() is included in MCONTROL.C, and

merely waits for the TXD register to be emptied of any current data.

It returns true if the register clears and false if it times out

waiting for them to clear.  The source for this function is also listed

below.

int SetIntMap( BYTE bIrq )

{

if( waitTXDE() ) {

CLI

outp( wBASEIO + HP_ICR , inp( wBASEIO + HP_ICR ) | HPICR_TREQ );

outp( wBASEIO + HP_IRQM , bIrq );

outp( wBASEIO + HP_ICR , inp( wBASEIO + HP_ICR ) & ~HPICR_TREQ );

STI

return(1);

}

return(0);

}

int waitTXDE()

{

int nTimeOutCount = 20000;

while(nTimeOutCount-- >= 0) {

if( inp(wBASEIO + HP_ISR) & HPISR_TXDE ) {

return(1);

}

}

return(0);

}

int Setup56kIRQ()

{

BYTE bGP;

if( SetIntMap( bIrq ) == 0 ) {

return(0);

}

if( nIRQValue > 7 ) {

bGP = nIRQValue + 0x68;

outp( PIC2 + 1 , ~( 1 << (nIRQValue - 8) & inp( PIC2 + 1 ) ) );

}

else {

bGP = nIRQValue + 8;

outp( PIC1 + 1 , ~( 1 <<  nIRQValue & inp( PIC1 + 1 ) ) );

}

nDSPSaveVect = _dos_getvect( bGP );

_dos_setvect( bGP , int_handler );

outp( wBASEIO + HP_ICR , inp( wBASEIO + HP_ICR ) | HPICR_RREQ );

return(1);

}

There is also a function Reset56kIRQ() included in MCONTROL.C

which restores the interrupt to its original value.  This should be

run in your exit code to insure system integrity.

Example #1 - AUXVOL.C and INVOL.C

We will now take the information we learned from above, and apply it to

a simple application.  This application, included on the disk, will

allow you to set the auxiliary input volume of the MultiSound.

Based on the preceding information we can write the entry and exit

code for our first example as follows.

#include <msnd_dsp.h>

#include <mcontrol.h>

BYTE bMem, bIrq, bBaseIO;

WORD wIRQValue, wBASEIO;

main()

{

bMem        = HPMEM_D000;   // defined in msnd_dsp.h

bIrq        = HPIRQ_10;     // defined in msnd_dsp.h

wIRQValue   = 10;

bBaseIO     = HPIO_3E0;     // defined in msnd_dsp.h

pMEM.dw.h   = 0xD000;

pMEM.dw.l   = 0x0000;

wBASEIO     = 0x3E0;

resetDSP();

ResetProteus();

uploadreb ("msndperm.reb");

uploadbin ("msndinit.bin");

initSMA();

Setup56kIRQ();

.

.

application code

.

.

Reset56kIRQ();

exit( 0 );

}

Here we have defined several global variables for use with the program.

The reason for defining these globally is that virtually all the

functions will need access to one or more of these values.  The

variable pMEM is a union defined in MCONTROL.H.  Its definition

follows:

union mem_tag {

BYTE far *p;

struct dw_tag {

WORD l;

WORD h;

} dw;

} pMEM;

This union allows us to access the SMA either as a far pointer, a WORD

offset, or the WORD describing the segment.  The constants HPMEM_D000,

HPIRQ_10, HPIO_3E0 are the register values for the associated

parameters and are defined in MSND_DSP.H.

Now we will add in the high level application code.  We will allow the

user to increase and decrease the volume using the '+' and '-' keys.

This will be done using a while loop based on a termination flag

( bTerminator ).  The flag will be set when the user hits the 'q' key,

and it will stop the program.  The application loop that we will

insert in the code will be as follows.

while( !bTerminator ) {

if( kbhit() ) {

switch ( getch() ) {

case 0x2B:

SMA->bAuxPotPosLeft++;

SMA->bAuxPotPosRight++;

SetAuxVolume();

printf("Aux Volume = 0x%X\n", SMA->bAuxPotPosLeft );

break;

case 0x2D:

SMA->bAuxPotPosLeft++;

SMA->bAuxPotPosRight++;

SetAuxVolume();

printf("Aux Volume = 0x%X\n", SMA->bAuxPotPosLeft );

break;

case 0x51:

case 0x71:

printf("Terminating\n");

bTerminator = 1;

break;

}

}

}

As can be seen when the user requests that the volume be increased, the

program responds by incrementing the SMA AUX volume variable, and then

calling the function SetAuxVolume().  This function must tell the DSP

that the value of the AUX volume has changed.  This function is

included in MCONTROL.C and is shown below.

void SetAuxVolume( void )

{

sendhostword( 0x00 , 0x00 , HDEXAR_AUX_SET_POTS );

SendDSPCommand( HDEX_AUX_REQ );

}

This function uses two other functions from MCONTROL.C.  The first one

loads the specific AUX request into the host port transmit registers.

The code for sendhostword is listed below.  First the function calls

waitTXDE to wait for the transmit data registers to clear any data

they currently contain.  Then it loads them with the AUX request.  If

the waitTXDE fails, the function returns false.

int sendhostword(BYTE bHi, BYTE bMid , BYTE bLow )

{

if( waitTXDE() ) {

outp( wBASEIO + HP_TXH , bHi );

outp( wBASEIO + HP_TXM , bMid );

outp( wBASEIO + HP_TXL , bLow );

return(1);

}

return(0);

}

The next function, sendDSPcommand(), sends the HDEX_AUX_REQ exception

to the DSP.  This interrupts the DSP and tells it that there is an AUX

request in the TXD registers.  The value of bCommand tells the DSP

which of its ISRs to execute.  The function waitHC() is similar to

waitTXDE(), only it is waiting for the DSP to acknowledge that it is

ready for another command.

int sendDSPcommand( BYTE bCommand )

{

if( waitHC() ) {

outp( wBASEIO + HP_CVR , bCommand );

return(1);

}

return(0);

}

Now we have constructed all the elements of our first MultiSound

application.  All that is necessary is to compile it.  The source for

this program is on the included disk.  Be sure enter the appropriate

hardware settings into your code before compiling (ie wBASEIO, pMEM.p,

and IRQ ).  If you are using Microsoft C compiler, you can use the

MAKEFILE included and run 'nmk' to compile the code.

When you run the program be sure that you have a line level audio

source hooked up to the auxiliary input.

To adjust the record input level we only have to change the

SetAuxVolume() function to SetInVolume(), and change the SMA variable

which is changed( SMA->bInPotPosLeft ).  SetInVolume() does the same

thing as SetAuxVolume() only it sends the HDEXAR_SET_IN_POTS to the TXD

registers.  The source code for this version is called INVOL.C and is

on the distribution disk.

CHAPTER 3 - Queues From Hell

Now that we have laid the foundation for programming the MultiSound,

the next step is to create an application which interacts with the DSP

in real time.  Before we can dot that we must have a firm grasp on how

the job queue system works in the MultiSound.

The SMA contains 5 job queues.  These are used to control the data for

digital audio recording, digital audio playback, MIDI in, MIDI out, and

DSP to host messages.  Each of these queues has a structure associated

with it, which is used for control of that particular queue.  This

structure is shown below, and is the foundation of the queue based

system by which the host and the DSP interact.

struct JobQueueStruct {

WORD wStart;

WORD wSize;

WORD wHead;

WORD wTail;

};

The 5 JobQueueStruct's are located in the SMA, since both the DSP and

the host will need access to them.  The wStart value will always be a

DSP based pointer to an associated data buffer, of size wSize.  Thus if

the structure was for MIDI in data, wStart would be a pointer to an SMA

buffer of size wSize where the DSP will write incoming MIDI data.  The

head and tail values are used to keep track of the current location in

the queue.  No matter which way the information is going, data is

always written to the tail of a queue, and read from the head.  So if

we were sending MIDI data, the host would write the data to the tail of

the queue, and the DSP would read it from the head.  On the other hand,

if we were receiving MIDI data, the DSP would write incoming data to

the tail, and the host would read that information off the head of the

queue.  Since any one queue will be used exclusively as either a

read or write data channel, only one element of the structure has to

be maintained by the host.  For data out queues, the host will

maintain the tail, and the DSP will maintain the head.  Likewise for

data in queues, the DSP will maintain the tail, and the host will

maintain the head.  The start and size parameters are constants

which will only be set at initialization time.  The names of the queue

structures used in this text are listed below along with who controls

each of the elements.

DAPQ -  Digital audio play job queue

Host owns wTail

DSP owns wHead

DARQ -  Digital audio record job queue

DSP owns wTail

Host owns wHead

MODQ -  MIDI out data job queue

Host owns wTail

DSP owns wHead

MIDQ -  MIDI in data job queue

DSP owns wTail

Host owns wHead

DSPQ -  DSP to host messages job queue

DSP owns wTail

Host owns wHead

For MIDI data and DSP messages the JobQueueStruct will point directly

to the associated buffer where the information is stored.  However for

digital audio play and record, the process is not as direct.  For these

the JobQueueStruct points to a DAQueueDataStruct, which contains the

pointer to the data buffer, as well as information on the type of audio

data contained in that buffer.  This Structure is shown below, and is

included in MCONTROL.H.

struct DAQueueDataStruct {

WORD wStart;

WORD wSize;

WORD wFormat;

WORD wSampleSize;

WORD wChannels;

WORD wSampleRate;

WORD wIntMsg;

WORD wFlags;

};

In this case of audio the job queue ( DAPQ or DARQ ) pointers refer to

a buffer of three of the data queue structures.  These structures in

turn have a pointer( wStart ) to the location of the actual audio data.

For the case of playing audio there are 3 buffers of 0x2400 bytes in

bank 0 of the SMA, each with an associated DAQueueDataStruct.  For

recording audio there are 3 buffers of 0x2000 bytes in bank 1 of the

SMA.  Again each of the record buffers has an associated

DAQueueDataStruct.

The other elements of the structure are used to control the audio

format.  The supported values are tabulated as follows.

wFormat 		-  1 = PCM

wSampleSize   	-  16 bit

8 bit

wChannels       -  1 = MONO

-  2 = STEREO

wSampleRate     -  11025 Hz

22050 Hz

44100 Hz

The remaining two values are for the hosts use.  Upon completion of a

job the DSP will interrupt the host, and place the value wIntMsg into

the DSP to host message queue.  The last variable wFlags can be used to

store any data that the user wants to have linked to a particular

buffer of audio data.

Now that we have an overview of the queuing mechanism used by

MultiSound we can start writing an application using them.  Although

the queue based system lends itself to audio multitasking( ie recording

audio and sending MIDI simultaneously )  the breadth of this manual

will only treat one audio task at a time.  It is left to the

adventerous programmer to undertake further development.

CHAPTER 4 -  Using Digital Audio

This chapter will walk you through the sample record and play

applications.  Before doing this, you may want to review the memory map

and the structure definitions in the reference section of this manual.

Doing this will make understanding the process much easier.

We will use the foundation code we wrote in chapter 2 to start us off

with.  The first thing we need to add to this is a message handling

system.  As the DSP completes tasks from its job queues, it will

interrupt the host and place messages, dependant on the particular task

completed, in the DSPQ data buffer( pointed to by DSPQ->wStart ).  So

when the host receives an interrupt from the DSP it will have read the

messages from the location pointed to by DSP->wHead.  Then it will have

to increment DSP->wHead and compare it to DSP->wTail.  This process

will continue until DSP->wHead is equal to DSP->wTail.

Now we will start to write the Interrupt Service Routine( ISR ).  This

manual assumes that you are familiar with writing and ISR.  If you are

not you may want to consult one of the references in FURTHER READING.

The following is a listing of the interrupt handler used by the demo

code in this tool kit.

void _interrupt _far int_handler()

{

if( bIntFlag != 1 ) {

bIntFlag = 1;

if( bCurrBank ) outp( wBASEIO + HP_BLKS , HPBLKSEL_0 );

inp( wBASEIO + HP_RXL );

if( nIRQValue > 7 ) outp( PIC2 , EOI );

outp( PIC1 , EOI );

while( DSPQ->wTail != DSPQ->wHead )  {

if( ++pwHostHead >= ( pwHostQueue + HOSTQ_SIZE ) )

pwHostHead = pwHostQueue;

*pwHostHead = *( pwDSPQData + DSPQ->wHead );

if( ++DSPQ->wHead > DSPQ->wSize ) DSPQ->wHead = 0x00;

}

if( bCurrBank ) outp( wBASEIO + HP_BLKS , HPBLKSEL_1 );

bIntFlag = 0;

}

}

The first thing we check on entry is the value of bIntFlag.  This is to

make sure we are not nested in the ISR.  If we are nested, we exit

immediately.  When this happens no data is lost.  This is because we

are allready reading the data off the DSP message data queue.  The DSP

will write the message into the queue and advance DSPQ->wTail, and the

host will continue to read messages until it reaches the tail.

The next value we check is bCurrBank.  This is a global variable which

tracks which SMA bank the host is currently using.  Since all the queue

structures are stored in SMA bank 0, we have to insure that we are

looking at that bank while inside the ISR.  We do this by writing, if

necessary, to the HP_BLKS register as explained in CHAPTER 1.

After we are definitely looking at the correct bank, we read the low

receive register of the DSP host port.

inp( wBASEIO + HP_RXL );

This acts as an acknowledge to the DSP that the port has been

read, enabling it to send more information if necessary.

After clearing the DSP and issuing the EOI( End Of Interrupt ) to the

PIC( Programmable Interrupt Controller ) we can start to read the data

off of the message queue.  For this sample code we have set up another

queue which we will refer to as pwHostQueue.  The ISR merely transfers

the new information in the DSP message queue into the pwHostQueue.

When the host exits from the ISR, it has its own queue of tasks to deal

with.

Once the host has read all the available information from the message

queue ( DSPQ->wHead = DSPQ->wTail ), it restores the previous state of

the machine.  To do this we restore the state of HP_BLKS, and reset

bIntFlag.

Now we have the framework for the application and the ISR to handle the

interrupts from the DSP.  All that remains is the processing of the DSP

messages which will be stored in pwHostQueue.

The main loop of our program is listed below.  Basicaly it keeps

checking for a difference between pwHostHead and pwHostTail until the

termination flag bTerminator is set or the user hits a key.  When the

DSP interrupts the host, the ISR will increase pwHostTail.  This will

cause our program to execute EvalDSPMessage.

while( !kbhit() && !bTerminator ) {

if( pwHostHead != pwHostTail ) {

if( ++pwHostTail >= (pwHostQueue + HOSTQ_SIZE ) ) {

pwHostTail = pwHostQueue;

printf("Wrapping Host Queue\n");

}

EvalDSPMessage( *pwHostTail );

}

}

All the possible messages that can be received from the DSP are defined

in the file MSND_DSP.H, and also listed in the reference section of

this manual.

These messages transmitted in TXH at interrupt time and also written to

the DSPQ data buffer.  For now we will only be concerned with

HIMT_RECORD_DONE and HIMT_DSP \ HIDSP_INT_RECORD_OVER.  However we will

write our EvaluateDSPMessage to include all messages to make the later

examples easier.  Our EvaluateDSPMessage is essentialy just a large

switch function with a case for every message we want to respond to.

The skelelton for the code follows.

void EvalDSPMessage( WORD wMessage )

{

switch ( HIBYTE( wMessage ) )

{

case HIMT_PLAY_DONE:

switch ( LOBYTE( wMessage ) )

{

// LOBYTE( wMessage ) will be either 0 , 1 , or 2

// depending on which buffer was completed

}

break;

case HIMT_DSP:

switch ( LOBYTE( wMessage ) )

{

case HIDSP_INT_PLAY_UNDER:

break;

case HIDSP_INT_RECORD_OVER:

break;

case HIDSP_MIDI_IN_OVER:

break;

case HIDSP_MIDI_OVERRUN_ERR:

break;

}

break;

case HIMT_RECORD_DONE:

switch ( LOBYTE( wMessage ) )

{

// LOBYTE( wMessage ) will be either 0 , 1 , or 2

// depending on which buffer was completed

}

break;

case HIMT_MIDI_IN_BYTE:

break;

}

}

Using this framework, we just have to fill in the code appropriate to

which messages we want to respond to.  For the HIDSP_INT_RECORD_OVER

message we will just increment a counter to track the number, if any,

of overflows that happened during the record.  The meat and potatoes of

this program is under the HIMT_RECORD_DONE message.

The first step is check the LOBYTE of the message to see if we are

receiving a second message for the same buffer.  If this is the case we

do not perform any processing.  Otherwise we set bLastBank to the

current buffer.

case HIMT_RECORD_DONE:

if( bLastBank != LOBYTE( wMessage ) ) {

bLastBank = LOBYTE( wMessage );

.

.

}

break;

The next step is to manage the job queue.  We have to store the tail

value, and increment it.  When using the digital audio queues

incrementing the tail means we have to make it point at the next

DAQueueDataStruct.  To do this we add the size of that structure to our

stored value.  If our new value is greater than the queue size, 3

elements, we will have to wrap the queue.  To do this we just set the

tail to zero.  As a precaution we make sure that queue is not wrapping

itself by comparing the head to the tail.  If they are equal we have to

wait until the DSP advances the head before we can advance the tail.

Then we send the DSP a HDEX_RECORD_START exception so it will check the

queue to see the update.  Our new code looks like this.

case HIMT_RECORD_DONE:

if( bLastBank != LOBYTE( wMessage ) ) {

bLastBank = LOBYTE( wMessage );

wTemp = DARQ->wTail + DARQ_STRUCT_SIZE;

if( wTemp > DARQ->wSize ) {

wTemp = 0;

}

while( wTemp == DARQ->wHead );

DARQ->wTail = wTemp;

SendDSPCommand( HDEX_RECORD_START );

.

.

}

break;

Now we have a buffer full of recorded digital audio data, and we have

to copy it to a local buffer, and then write it to disk.  First we get

a pointer to the buffer where the data is located based on the LOBYTE

of the DSP message sent to us.  Since the record buffers are located in

the SMA bank 1, getting the data is a little tricky.  First we change

the bank the host is looking at by writing to the HP_BLKS register.

During this transition we can't have any interrupts so we disable them

( CLI ) going into write, and then re-enable them when leaving ( STI ).

Once we are looking at the correct bank, we can copy the data to our

local buffer.  Then we restore HP_BLKS so the host is looking at bank

0.  We must disable interrupts for this transition as well.  At last we

write the data to disk.  The code now looks like this.

case HIMT_RECORD_DONE:

if( bLastBank != LOBYTE( wMessage ) ) {

bLastBank = LOBYTE( wMessage );

wTemp = DARQ->wTail + DARQ_STRUCT_SIZE;

if( wTemp > DARQ->wSize ) {

wTemp = 0;

}

while( wTemp == DARQ->wHead );   // Wait if we are going to wrap the queue

DARQ->wTail = wTemp;

SendDSPCommand( HDEX_RECORD_START );

pbCurRecordBuff = ( BYTE far *)( pMEM.p + bLastBank * DAR_BUFF_SIZE );

CLI;

outp( wBASEIO + HP_BLKS , HPBLKSEL_1 );

bCurrBank = 1;

STI;

_fmemcpy( pbHostBuffer , pbCurRecordBuff , DAR_BUFF_SIZE );

CLI;

outp( wBASEIO + HP_BLKS , HPBLKSEL_0 );

bCurrBank = 0;

STI;

fwrite( pbHostBuffer , sizeof( BYTE ) , DAR_BUFF_SIZE, f_point );

}

break;

The only remaining task is taking care of the file opening, pre-loading

the queue and file closing.  Since these are straight forward it is

left to the user to look at the final source code, included on the

distribution disk, to see how this is accomplished.  The only

explanation left is the calibration of the analog to digital

converters.  The function CalibrateAD( WORD wSampleRate ) is provided

in MCONTROL.C to accomplish this.  The source for CalibrateAD is shown

below.

void CalibrateAD( WORD wSampleRate )

{

BYTE bTime;

SMA->wCalFreqAtoD = wSampleRate;

sendhostword( 0x00 , 0x00 , HDEXAR_CAL_A_TO_D );

SendDSPCommand( HDEX_AUX_REQ );

// The actual time for the calibration is equal to

// 4096 / SAMPLE_RATE

// multiply by 100 to get bTime as # of hundreths of seconds

bTime = 100 * 4096 / SAMPLE_RATE;

GetDelayTimefor( bTime );

}

?? put something in here about preparing the DAQueueDataStruct ??

This function takes the recording sample rate desired as a parameter.

It calculates the calibration time based on this value.  Then it sets

the sample rate in the SMA variable SMA->wCalFreqAtoD.  Next it puts

the particular AUX request into TXL, and sends the HDEX_AUX_REQ

exception to the DSP.  It finishes off by calling GetTimeDelay, a

function which waits the specified number of hundreths of a second and

then returns.

Now we have a fully functional recording program.  The sample rate,

bits per sample, and number of channels can be set using the defines in

the beginning of the program.  Now we can start writing a program which

will play the audio.

Playing Audio

To play audio is almost identicle to recording the audio.  We can use

the same framework we used for recording.  The only Major change

we have to make is the message response in EvalDSPMessage.  Playing is

actually easier since the play data buffers are in the same SMA bank as

the queues, so we do not have to change our SMA window to access the

data.

The distribution disk contains the source in PLAY.C.  If you take a

look at it you will see that main routine is basically the same as in

RECORD.C.  Instead of setting up the DARQ structure, we setup the DAPQ

structure, as well as the associated DAQueueDataStructs.  Also you will

note that we don't have to calibrate the converters.  We change the

entry code to pre load the buffers with audio data from our file.  Then

we only have to add a case to EvalDSPMessage.

In PLAY.C when we get a bank empty message, we have to load a new

buffer of information into the SMA play data buffer from the disk.  As

with recording the first step is to make sure we are not getting a

repeted message.  LOBYTE( wMessage ) will contain the buffer number

which the DSP has just finished playing.

case HIMT_PLAY_DONE:

if( bLastBank != LOBYTE( wMessage ) ) {

bLastBank = LOBYTE( wMessage );

.

.

.

}

break;

To reduce the chance of data underflows, we allways want to have all

three SMA buffers filled with data.  You will notice in the entry code

that we preload all three buffers.  However we only set the

DAPQ as if two were loaded.  So when we get an HIMT_PLAY_DONE message

we just advance DAPQ->wTail, and the data is allready there.  Then we

send the DSP exception so it will look at the queue.  Before leaving we

must advance our own pointers, and read the next buffer from the disk.

So when we get the next HIMT_PLAY_DONE message our data is allready

loaded.  Our final play done section now looks like this.

case HIMT_PLAY_DONE:

if( bLastBank != LOBYTE( wMessage ) ) {

bLastBank = LOBYTE( wMessage );

CurDAQD->wSize = (WORD)lReadSize;

if((DAPQ->wTail += PCTODSP_OFFSET(DAPQ_STRUCT_SIZE)) > DAPQ->wSize ) {

DAPQ->wTail = 0;

}

SendDSPCommand( HDEX_PLAY_START );

if( ++CurDAQD > (LPDAQD)( pMEM.p + DAPQ_DATA_BUFF + 2*DAPQ_STRUCT_SIZE ) )

CurDAQD = (LPDAQD)( pMEM.p + DAPQ_DATA_BUFF );

if( ( pbCurPlayBuff += DAP_BUFF_SIZE ) > (pMEM.p + 2*DAP_BUFF_SIZE))

pbCurPlayBuff = pMEM.p;

lReadSize = fread( pbCurPlayBuff , sizeof( BYTE ) , DAP_BUFF_SIZE, f_point );

}

break;

Now we have the ability to play and record audio through the

MultiSound.  As you play around with this code, you can use the

SMA->wCurrPlayVolLeft and SMA->wCurrPlayVolRight variables to

adjust the out put volume of the audio.

This completes our coverage of digital audio.  The reference section

should provide you with all the information you need to program the

card once you understand the basic concepts explained in this chapter.

It also provides a list of references for further reading on digital

audio and PC programming.

CHAPTER 5 - Using MIDI

The MultiSound has a Proteus 1/XR MIDI sound module on board.  This

section will explain how to access the Proteus as well as the external

MIDI port.  If you are not familiar with MIDI you should consult one of

the references at the back of this book.  This manual assumes the

reader is knowledgeable on the subject.

Like our work with audio on the MultiSound, MIDI is controlled via a

queueing system.  The MultiSound has two MIDI data buffers in the SMA.

One is for transmittiing MIDI data and the other is for receiving.  To

send MIDI, place the data you want to transmit into the MIDI out buffer

at the tail, offset MODQ->wTail, and advance the tail so the DSP knows

there is data in the buffer.  Likewise when there is MIDI data coming

in, the DSP will interrupt the host.  When the host receives a

HIMT_MIDI_IN_BYTE message it reads the MIDI byte from the head of the

MIDI in data buffer, offset MIDQ->wHead.  Several functions are

contained in MCONTROL.C which ease the manipulation of MIDI data.

?? MIDI hardware implementation ??

Our first example will show how to start and stop the Proteus demo

sequence.  The important concepts to learn in this example are how to

send MIDI data and how to route that MIDI data to the desired location.

Starting and stopping the demo is accomplished by sending the proteus a

sequence of system exclusive messages.  We will start with the code

framework used in chapter 2.  Since we are only transmitting data it

will not be necessary to respond to messages from the DSP.  The

convention in this manual is to terminate MIDI strings with a 0xFF.

This information is not sent over MIDI, it is merely used to signify to

the host that there is no more data to send.

The system exclusive MIDI sequences for starting and stopping the

proteus are included in PROSTART.H.  All we need to do is transmit

these strings out through the MIDI port.  But first we will set the

MIDI out port to the Proteus.  The function SetMidiOutPort in

MCONTROL.C accomplishes this.  In our main routine all we need to do is

put in the following line.

SetMidiOutPort( MOP_PROTEUS );

The constant MOP_PROTEUS is actually a bit mask to select the Proteus

for output.  The other associated bit masks are listed below.

#define  MOP_PROTEUS    0x10

#define  MOP_EXTOUT     0x32

#define  MOP_EXTTHRU    0x02

#define  MOP_OUTMASK    0x01

You can easily select any of the three possible MIDI outs using these

constants.  MOP_OUTMASK is used to inside of SetMidiOutPort to mask out

the MIDI in setting so it remains unaffected.  SetMidiOutPort gets the

current out port in SMA->wCurrMIDIIOPatch, and masks the input values

out.  Then  this new value is ORd with the selected ports mask.  Then

the new value is written back to SMA->wCurrMIDIIOPatch.  The source

for this function is shown below.  To make the actual connection change

we have to put the patch change command into the MIDI out data buffer.

This is done by first placing a 0xFE00 into the queue, and then placing

the new patch value into the queue.  The function MODPortWrite actually

does the writing of the data into the data buffer.  This scheme for

patch changing insures that all transmitted data will go to the correct

place when a patch change occurs.

void SetMidiOutPort( BYTE bNewMOP )

{

BYTE bMASK;

bMASK = ( MOP_OUTMASK & LOBYTE( SMA->wCurrMIDIIOPatch ) );

bNewMOP = ( bNewMOP | bMASK );

SMA->wCurrMIDIIOPatch = MAKEWORD( bNewMOP , 0x00 );

MODPortWrite( 0xFE00 );       // Flag to make sure Current Data is sent

// out before changing patch

MODPortWrite( MAKEWORD( bNewMOP , 0xFF ) );

}

Now that we know what we have to send we just need to know how to

send it.  For this task we will write the function MODPortWrite.  This

will have to take the data sent to it, write it into the MIDI out

queue, and then update the MODQ structure so the DSP is aware of the

data.  The first thing the code must do is check to see if the queue is

empty.  If it is we set our bIsEmpty flag.  If the queue is empty we

have to restart the DSP by sending it the HDEX_MIDI_OUT_DATA exception.

This is done after we write the data into the MIDI out buffer.  Next we

advance a copy of MODQ->wTail and check to see if we are wrapping at

the end of the buffer.  Once we have the next tail offset in wTail, we

check this value against MODQ->wHead.  This protects from the case were

the buffer is full and we are about to write over the head of the

queue.  Once we are sure we won't over run the queue we can perform the

acutual writing of the data into the buffer, and update MODQ->wTail.

The code for this function is shown below.

void MODPortWrite( WORD wMIDIData )

{

WORD wTail;

BYTE bIsEmpty;

if( MODQ->wHead == MODQ->wTail ) bIsEmpty = 1;

wTail = MODQ->wTail;

if( ++wTail > MODQ->wSize ) wTail = 0;

while( wTail == MODQ->wHead );     // Wait if Buffer is full

*( pwMODQData + MODQ->wTail ) = wMIDIData;

MODQ->wTail = wTail;

if( bIsEmpty ) SendDSPCommand( HDEX_MIDI_OUT_START );

}

Now that we have the correct MIDI out patch set all we have to do is

dump out the sequence to start the demo.  This sequence is in PRODEMO.H

and is pointed to by ProDemoMidiSequence.  We will use the same

function we just went over , MODPortWrite, and just step through

ProDemoMidiSequence writing the data to the queue.  When we are done

with the string, the proteus demo will start to play.  The program

will then wait for the user to hit a key, and then send the sequence

to stop the demo.  The code for this is listed below.

main()

{

BYTE *szDataString;

BYTE bMidiOutData;

bIntFlag    = 0;

bMem        = HPMEM_D000;

bIrq        = HPIRQ_10;

nIRQValue   = 10;

pMEM.dw.h   = 0xD000;

pMEM.dw.l   = 0x0000;

wBASEIO     = 0x3E0;

ResetProteus();

resetdsp();

uploadreb ("msndperm.reb");

uploadbin ("msndinit.bin");

initSMA();

Setup56kIRQ();

SetMidiOutPort( MOP_PROTEUS );

szDataString = ProDemoMidiSequence;

while ((bMidiOutData = *szDataString++) != 0xFF) {

MODPortWrite ( MAKEWORD( bMidiOutData , 0x00 ) );

if( *szDataString == 0xF7 ) {

GetDelayTime( TIME_PRO_SYSEX );

}

}

printf("Proteus demo is playing\nHit Any Key To Stop...\n");

while( !kbhit() );

getch();

szDataString = ProDemoStop;

while ((bMidiOutData = *szDataString++) != 0xFF) {

MODPortWrite ( MAKEWORD( bMidiOutData , 0x00 ) );

if( *szDataString == 0xF7 ) {

GetDelayTime( TIME_PRO_SYSEX );

}

}

Reset56kIRQ();

exit(0);

}

You will notice that we run the function GetDelayTime after every 0xF7

we send to the Proteus.  This is because 0xF7 allways signifies the end

of a system exclusive message.  After the Proteus receives a full

system exclusive message it requires approximately 30 milliseconds

before it can accept any valid data.  The function GetDelayTime is

included in MCONTROL.C and when executed does not return until the

passed number of 1/100ths seconds have passed.

The distribution disk also has the source for a program called

PRESETS.C.  This program operates the same way as PRODEMO.C  the only

difference is that the data is read from a file.  PRESETS.C can be used

to load the general midi patch set up to the proteus under DOS.

In our next example we look at receiving MIDI data from an external

source.  In the case of receiving MIDI data we will have to process

messages from the DSP.  So the framework for the code will look like

our digital audio skeleton.  We will have the host queue, and wait for

the ISR to dump some data into it.  When this happens we will respond

by transmitting that data back out to the Proteus.

Setting the MIDI in port works exactly like setting the MIDI out port,

only the associated masks are different.  The function SetMidiInPort is

included in MCONTROL.C.  The available masks for the input port are

listed below.

#define  MIP_EXTIN      0x01

#define  MIP_PROTEUS    0x00

#define  MIP_INMASK     0x32

As mentioned above our framework for this program remains the same as

before.  When we enter we have to set both of the desired MIDI patches,

and then send the DSP commands to initiate its MIDI IO routines.  Once

the preliminary set stuff is complete our main loop looks just like the

play and record routines.  Here we wait for the ISR to dump information

into the host queue.  When data is received we execute EvalDSPMessage

which as before is just a large switch process.  Under the particular

case of HIMT_MIDI_IN_BYTE, we will just call MidiInData() the function

which will manage the queue.  The source for the main routine is listed

below.

SetMidiOutPort( MOP_PROTEUS );

SetMidiInPort( MIP_EXTIN );

SendDSPCommand( HDEX_MIDI_IN_START );

SendDSPCommand( HDEX_MIDI_OUT_START );

while( !kbhit() ) {

if( pwHostTail != pwHostHead ) {

if( ++pwHostHead >= ( pwHostQueue + HOSTQ_SIZE ) ) {

pwHostHead = pwHostQueue;

printf("Wrapping Host Queue\n");

}

EvalDSPMessage( *pwHostHead );

}

}

SendDSPCommand( HDEX_MIDI_IN_STOP );

The function MidiInData continues to call MIDPortRead untill the MIDI

in data buffer is empty.  Every MIDI byte that it reads in is sent out

using MODPortWrite.  When a key is pressed on an external keyboard, the

DSP will read the MIDI in data and write it into the MIDI in buffer,

and interrupt the host.  The host will then write that same byte out to

the proteus.  The functions MidiInData and MIDPortRead are listed

below.

void MidiInData()

{

WORD wMidiData;

while( MIDQ->wHead != MIDQ->wTail ) {

wMidiData = MIDPortRead();

if( HIBYTE( wMidiData ) == 0x00 ) {

MODPortWrite( wMidiData );

}

printf( "%.2X " , LOBYTE( wMidiData ) );

}

}

WORD MIDPortRead( void )

{

WORD wRetVal , wHead;

wHead = MIDQ->wHead;

if( ++wHead > MIDQ->wSize ) wHead = 0;

wRetVal = *( pwMIDQData + MIDQ->wHead );

MIDQ->wHead = wHead;

return( wRetVal );

}

MIDPortRead operates similar to MODPortWrite.  However instead of

writing to the tail of the MIDI out buffer, here we are reading

from the head of the MIDI in buffer.