QSTART.DOC

Kingston Technology Company

QStart Information File

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Version 1.3, July 16, 1998

Copyright (C) 1998 Kingston Technology Company.

All rights reserved.

QStart Installation

QStart is designed to make the process of installing your Kingston

Ethernet adapter and network driver software as easy as possible.

Regardless of how much networking experience you have, QStart will

help you get up and running fast. QStart is a DOS program with a

Windows like graphical interface. Menu options can be selected using

either your keyboard or mouse. If you are using a mouse, you simply

point and click to highlight and select options; using your keyboard,

press [Tab] to highlight an option and [Enter] to select it.

There is also a shortcut key option that allows you to quickly select

QStart menu items. The first letter of each menu item is underlined.

The underline indicates the letter can be used as a shortcut key to

select that particular menu item. To select and activate any of the

menu selections, simply press and hold the [Alt] key and then press

the underlined letter on your keyboard. You do not need to press the

[Enter] key.

QStart features both automatic and manual installation options. By

choosing one of the automatic options, QStart will test your new

EtheRx PCI adapter and then configure the adapter to your

particular network by loading the appropriate network drivers.

The Custom (manual) option is designed for those who want more control

over the configuration of their network software and hardware. The

Custom Install option includes many of the same features found in the

two automatic options. However, unlike the other options, Custom

Install includes two diagnostics tests and on-line document viewing.

To help guide you through the installation process, each QStart screen

includes a help button that will display additional information about

options available for that screen.

Note:

1.

In order to run QStart, you need to have 450K Bytes of free

conventional memory under DOS.

2.

To use QStart under Windows 95/98, you need to run the

program under Windows' MS_DOS mode. In order to go to

MS_DOS mode, click on 'Start', then click on 'Shut Down'.

Select 'Restart the computer in MS_DOS mode?'.

3.

If you are using Windows 98, you may need to restart the

system and get to MS-DOS mode by using the F8 key.  This

is because Windows 98 may cause a BIOS configuration problem.

To run QStart, insert this Drivers Diskette into your PC's A: drive,

then type:

C:\>A:QSTART [Enter]

QStart offers several command line options. The syntax

for using QStart is as follows:  QStart [options...]

The following is a list of valid options:

EDITOR [fn]

This brings up a text editor with optional file name [fn].

GRAPHICS

Forces the program to use graphics mode.

HELP

The HELP or ? option displays this screen.

TEXT

This option forces the program to operate in text mode.

WIZARD

Starts the Help Wizard which displays I/O and IRQ information.

KEYSTROKE [fn]

Set macro keystroke file name (Default=QSTART.QSM).

(See the section MACROS below for more information

about creating and using macros.)

LAUNCH

Start the macro keystroke file playing immediately when

QStart loads.

REALTIME

Macro keystroke file will play back at the same speed as

recorded.

The first letter of the option may be used instead of the entire keyword.

For example: 'QSTART TEXT ALL' would be the same as 'QSTART T A'.

If you have more than one Kingston EtheRx PCI Ethernet adapter

installed in your computer, the program will detect and display each

adapter. From the list of displayed adapters, you must choose the

adapter you wish to install.

Next, the QStart main menu will be displayed on your screen. This menu

gives you three different options for configuring your network adapter

and software. The first two are automatic options - i.e., your

hardware and software will be automatically configured for you. The

third selection is a Custom (manual) option that allows you more

control over how the hardware and software are configured. Which

option should you choose? The option you select depends on your needs.

Starting below, the features of all three options are outlined.

ODI INSTALL

ODI Install is a fully automatic option designed to create a conflict

free configuration for your Ethernet adapter and configure your PC as

a Novell Netware DOS ODI client. When you select this option, your

Ethernet adapter is automatically tested. The Adapter Test does

two things: A quick series of functional tests are performed.

If an error occurs during any of the individual tests, the test

will halt and the error type will be displayed on the screen. If

you fail any of the tests, you should make sure both the adapter

and your network cabling are properly installed.  Next, QStart

uses it's built in intelligence to autodetect the Frame Type

and fill in settings that will work on your network.

After the hardware has been configured and tested, the program will

prompt you to enter the Netware Version, Preferred Server, User Name,

and Default Frame Type.

The Netware Version field displays one choice; to see additional

choices, press the down arrow and additional choices will be

displayed. From the list of choices, simply select the desired

Netware version.

If you are unsure about any of the settings on the screen, using

the defaults filled in by the program and selecting 'Continue'

will usually work properly.  QStart uses it's built in intelligence

to autodetect and fill in settings that will work on your network.

Once you have made all your selections, select Continue and the

Novell ODI client software will be configured and installed on your

PC. As a final step, the program will prompt you that a copy of your

previous CONFIG.SYS and AUTOEXEC.BAT files were saved with .KTC

extensions. If your AUTOEXEC.BAT or CONFIG.SYS files already exist

with .KTC extensions, they will be assigned a .K01, .K02, .K03,

(etc.) extension as required. If there is a previous version of the

DOS ODI driver installed, the previous NET.CFG and STARTNET.BAT will

be saved with .KTC extensions. If your NET.CFG or STARTNET.BAT already

exist with .KTC extensions, they will be assigned a .K01, .K02, (etc.)

extension as required. At this point, you can select Reset and your

computer will reset, allowing you to log onto the network; or you can

select Exit and you will be returned to the DOS prompt. To return to

the Main Menu, select Previous.  If you want to add additional edits

or comments to your CONFIG.SYS or AUTOEXEC.BAT files, select the Edit

Config. button.  If you select Exit or Previous, your network

configuration will not be enabled until the next time the computer

is reset.

GENERIC INSTALL

This option allows you to choose from a wide variety of network

operating systems. Once you select your particular NOS, QStart

will then copy the appropriate network driver and related software

to your system.

The Generic Install option will automatically test your Ethernet

adapter hardware configuration. After the hardware configuration

test is complete, the program will display a list

of available Network Operating System drivers. The program also

allows you to change the name of the default destination directory -

i.e., the name of the directory where the network driver and

associated files will be copied onto your hard disk drive.

After you have made all of your selections, select 'Copy' to copy your

network software or select 'Cancel' to abort the installation process.

At this point, you can also select 'View Doc' to view or print the

installation document for the driver of your choice.

CUSTOM INSTALL

Custom Install includes the following options: Basic Test,

Transmit/Receive Test, Driver Installation, and Document View.

-Basic Test

The Basic Test performs a quick series of diagnostics to insure

your network adapter is functional.

-Transmit/Receive Test

The Transmit/Receive Test is a more comprehensive test which

tests network communication with another PC running the same

test on your network.  The Transmit/Receive Test option requires

two PCs and both PCs must be connected to the same network.

-Driver Installation

The Driver Installation option allows you to select and copy the

desired Network Operating System driver and related software to

your PC.  Besides Novell Netware DOS ODI software, there are a

number of other popular Network Operating System software drivers

to choose from.  The Driver Installation option also allows you

to view and print installation instructions for any of the

Network Operating Systems included.

-Document View

The Document View option allows you to view or print any of the

available on-line documentation.  This documentation includes a

README.DOC file describing the contents of the Kingston Ethernet

Adapter Driver Diskette, a QSTART.DOC file (which is the file

you are reading now) that outlines the capabilities of the QStart

installation program, as well as other documentation designed to

assist you in your network configuration and installation process.

Kingston Technology Company, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.

MACROS

QStart's macro feature allows you to record keystrokes and mouse

clicks for playback at a later time.

If you are performing the same tasks for many adapters, this feature

can save you much time by allowing you to automate the process.

Creating a Macro:

-- Load QStart.

-- Hold down the <Ctrl> key and press <F1>.  This tells QStart that

you want to record a macro.

-- Select 'Continue' to begin recording when QStart prompts you about

macro recording.

-- Enter the keystrokes and mouse clicks you want included in your

macro.  These events will be recorded to disk.  (The default

macro file name is QSTART.QSM.  However, you can change this using

QStart's KEYSTROKE command line option.)

-- When you have finished entering all the keystrokes and mouse clicks

you want included in your macro, hold down the <Ctrl> key and press

<F1> to stop recording.

Creating Macros With Pauses:

QStart's macro pause feature allows you to create a macro that will

pause until the user tells the macro to resume playing.

This is very useful if you want the macro to pause for the user to

enter some information or perform some operation.

While recording a macro, you can place a pause anywhere you want in

your macro by holding down the <Ctrl> key and pressing <F3>.  Hold

down the <Ctrl> key and press <F2> to resume recording.  This pause

will then show up during playback.

Playing Back The Macro:

Hold down the <Ctrl> key and press <F2> to start your macro playing.

You can also use the command line option LAUNCH to start the macro

playing immediately when QStart loads.

Macro Command Line Options:

QStart has several command line options that affect macro playback and

recording.  These are as follows:

KEYSTROKE [fn]

Set macro keystroke file name (Default=QSTART.QSM).

LAUNCH

Start the macro keystroke file playing immediately when

QStart loads.

REALTIME

Macro keystroke file will play back at the same speed as

recorded.

Summary of Macro Contol Keys:

<Ctrl-F1> = Record Start/Stop

<Ctrl-F2> = Play/Resume

<Ctrl-F3> = Pause

Macro Tips:

A. Using keystrokes instead of mouse movement and clicks saves disk space.

B. Using the <Alt> <letter> shortcut keys is the most effective.