PrintGL 1.60 Plotfile Commands and Program Interface

(c) Copyright Ravitz Software Inc. 1990,1996

Ravitz Software Inc.                BBS/fax               606-268-0577

PO Box 25068

Lexington, KY  40524-5068           Compuserve  Cary Ravitz [70431,32]

USA                                 Internet   70431.32@compuserve.com

------------------------------ Contents ------------------------------

PrintGL Initial Information File ......................... PRINTGL.TXT

PrintGL Documentation File                                 PRINTGL.DOC

Introduction ....................................................... 3

HP-GL Commands                                                       3

HP-GL/2 Commands .................................................. 11

ADI Commands                                                        13

HP-GL Character Sets .............................................. 14

Program Interface                                                   16

p-3

Introduction ---------------------------------------------------------

This  document  includes  a  description  of  the  subset  of  HP-GL

(Hewlett-Packard  Graphics  Language),  HP-GL/2,  and ADI  (Autodesk

Device  Interface) that  is  supported by PrintGL.  If you are using

PrintGL  to work with commercial  graphics  software,  then you will

probably not need  this information.  If you  are  writing  graphics

software you will  find here  the  information needed  to  make your

output compatible with PrintGL.

The program interface to PrintGL is also documented here.

HP-GL Commands -------------------------------------------------------

PrintGL's HP-GL specification is taken from the  programming manuals

for the HP 7475A, ColorPro (7440/17440), and 7550A.  The 7550 manual

is  the  best reference for HP-GL - HP  manual 07550-90001.  PrintGL

supports  the 7475, and 7440/17440 command sets except for character

set 8 (Katakana), digitize commands, output commands, the error mask

command, and some device control  commands.  Supported commands  are

listed below.  Commands that are not applicable to printing, such as

pen velocity, are  considered supported  but are completely ignored.

Commands that are not supported are flagged and then ignored.

In addition, PG,  AF, ES, LO, the enhanced IW command, character set

5, and proportional spaced fonts, from the 7550, are supported.  And

forty pens are allowed instead of eight.

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (026) is interpreted as an end of file mark.

AA x_center , y_center , arc_angle [, chord_angle]

Arc absolute - AA moves  the pen through  an arc,  starting at the

current position, with the pen in the current up/down state.

AF

Advance full page -  AF  starts  a new plot.  PrintGL starts every

page with the pen up at 0,0.

AP

Automatic pen pickup - IGNORED

AR x_center_delta , y_center_delta , arc_angle [, chord_angle]

Arc  relative  - AR moves the pen through an arc, starting at  the

current position, with the pen in the current up/down state.

AS

Acceleration select - IGNORED

p-4

CA character_set

Designate alternate character set - See HP-GL Character Sets for a

list of character sets.

CI radius [, chord_angle]

Circle - CI draws a circle centered at the  current position.  The

current up/down state and position are unchanged by a CI command.

CP [spaces , lines]

Character plot  - CP moves the pen by character spaces (+ is along

the text)  and lines  (+ is up), using  the current up/down state.

Using no parameters is the same as a carriage return/line feed.

CS character_set

Designate standard character set - See HP-GL Character Sets for  a

list of character sets.

CV

Curved line generator - IGNORED

DF

Default values -  This  sets  default  values  for  the  following

commands: CA, CS,  DR, DT, ES, FT, IW, LO, LT, PA, PT, SC, SM, SL,

SR, SS, TL.

DI [run , rise]

Absolute  character direction - DI sets the baseline direction for

labels. The default is 1,0.

DR [run , rise]

Relative character direction - DR sets the  baseline direction for

labels. Run and rise are a percentage of P2X-P1X and P2Y-P1Y.  The

default is 1,0.

DTt

Define label terminator - The label terminator ends an LB command.

The default is ASCII 003.

EA x_opposite_corner , y_opposite_corner

Edge  rectangle  absolute  - EA draws a rectangle  defined by  the

current position and the parameters.  The pen position and up/down

state are unchanged.

EC

Enable cutter - IGNORED

p-5

EP

Edge polygon - EP draws the current polygon edge.  Any pen up/down

used in creating the polygon is respected in drawing the edge.

ER x_opposite_corner_delta , y_opposite_corner_delta

Edge  rectangle relative  -  ER  draws  a rectangle  with  corners

defined by  the current position  and the command parameters.  The

current pen position and up/down state are unchanged.

ES [ space [, line]]

Extra space  - ES adds to the  space between characters and lines.

The  units  are in  character  cells  (1.5*width,  2*height).  The

allowed range is -1 to 1. -.3333 causes characters to touch.

EW radius , start_angle , sweep_angle [, chord_angle]

Edge  wedge - EW draws a wedge centered  at the  current position.

The current pen position and up/down state are unchanged.

FP

Fill polygon - FP fills the current polygon.  Pen up/down commands

used in creating the polygon are ignored.

There is a limit of 4000 fill line segments per polygon.

FS

Force select - IGNORED

FT [type [, spacing [, angle]]]

Fill type - These are the fill types.  Type 2 is identical to type

1 for PrintGL.

1  solid lines spaced at half the pen thickness (see PT command)

2  unidirectional solid lines spaced at half the pen thickness

3  lines at the specified spacing

4  cross hatched lines at the specified spacing

5  type ignored

A spacing  parameter  of 0 sets  the  spacing  to 1%  of the P1-P2

diagonal. Spacing is in x axis units (this is only relevant if the

axes have different scaling).  PrintGL supports any  integer angle

for fill. Cross hatched lines are at angle + 90.

GM

Graphics Memory - IGNORED

IN

Initialize

p-6

IP [p1x , p1y [, p2x , p2y]]

Sets scaling points P1 and P2 - IP sets the scaling points (P1 and

P2) in plotter units.  P1  and P2 are used by several commands  to

help with size independent plots.

IW [x_lower_left , y_lower_left , x_upper_right , y_upper_right]

Input window - IW sets a clipping window in plotter units. With no

parameters, the clipping window is turned off.

If  PrintGL's  /YE switch is  set and user  scaling  is on then IW

parameters are in user units.  In this case the window is rescaled

with an IP command, but any SC command locks the current window.

LB..text..terminator

Label - Labels use the current character set, based on CA, CS, SA,

and SS commands. See HP-GL Character Sets for font specifics.

LO [ position_number ]

Label origin -  LO  sets  the  label  origins,  shown  below.  For

positions 4..9  and  14..19  the  label buffer is  limited  to 150

characters. The default is 1.

13                     16                      19

3......  .......  ...6...  .......  ......9

.     .  .     .  .     .  .     .  .     .

12 2     .  .     .  .  5  .  .     .  .     8 18

.     .  .     .  .     .  .     .  .     .

1......  .......  ...4...  .......  ......7

11                     14                      17

LT [pattern_number [, pattern_length]]

Line  type  - PrintGL's line type patterns are close  to, but  not

identical  to HP's.  Pattern  length is a  percentage of the P1-P2

diagonal with a default of 4%  and a minimum of 4 dots for types 1

and 2 and 16 dots for other types.  Negative (adaptive) line types

from the HP 7550 are supported.

no parameter  solid            1  .               .

2  ________        ________

0             dots at          3  ____________    ____________

endpoints        4  ____________  . ____________  .

only             5  __________  __  __________  __

6  ______  __  __  ______  __  __

7  __________  . . __________  . .

8  ______  . __  . ______  . __  .

PA [x , y] [, x , y] ...

Plot absolute - PA sets absolute mode and optionally moves the pen

in the current up/down state.

p-7

PD [x , y] [, x , y] ...

Pen down - PD sets the pen  down and optionally moves it with  the

current absolute/relative mode.

PG [i]

New page - PG starts a new plot. The parameter is ignored. PrintGL

starts every page with the pen up at 0,0.

PM i

Polygon mode - PM  0 starts polygon mode.  The current position is

the first point in the polygon.

PM 1 starts  a new  subpolygon.  The  position after the next  pen

movement is the first point in the subpolygon.  The first pen move

of a subpolygon is done with the pen up, regardless of the current

up/down status.

PM 2 ends polygon mode.

PM  1 and PM 2 close  the current subpolygon with  the current pen

up/down state. If used without first starting polygon mode (PM 0),

PrintGL adds an implied PM 0 before processing the PM 1 or PM 2.

The following  commands are accepted in polygon  mode: PM, PA, PR,

PU, PD,  AA,  AR,  CI  (circles  are  automatically  prefixed  and

suffixed by PM 1), and IN.

The polygon buffer is limited to 1000 data points.

PR [x_delta , y_delta] [, x_delta , y_delta] ...

Plot relative - PR sets relative mode and optionally moves the pen

in the current up/down state.

PS

Paper size - IGNORED

PrintGL ignores PS. The paper size must be set with the /L option.

PT [thickness]

Pen  thickness - PT specifies twice the line spacing used in solid

fills and may be from 0 to 5.0 mm.  It is reset to .3 mm with each

SP command.

PU [x,y] [,x,y] ...

Pen  up  - PU  raises  the pen and optionally  moves  it with  the

current absolute/relative mode.

p-8

RA x_opposite_corner , y_opposite_corner

Shade  rectangle  absolute  -  RA  fills  a rectangle with corners

defined  by the current position and the  command parameters.  The

current  pen  position  and  up/down  state  are  unchanged.   The

rectangle edge is not drawn.

RO [angle]

Rotate coordinate  system - RO or RO0  sets up a normal coordinate

system.  RO90, the only other allowed value, sets up a system that

is rotated 90 degrees.  The type of rotation depends on  the paper

size.  This command  does  not change the scaling points, so an IP

command is useful after an RO.

The HP 7475 specifies two different meanings for RO90 depending on

the  paper  size (A/A4 or B/A3).  PrintGL  decides  which  type of

rotation to do based on the sum of  the  print  window  width  and

height.  If this sum  is  greater than 24 inches  then large paper

rotation is used. Otherwise, small paper rotation is used.

Small paper rotation:              Large paper rotation:

RO 0             RO 90             RO 0             RO 90

y --------    0,0 -------- y     0,0 -------- y    y --------

|        |      |        |         |        |      |        |

|        |      |        |         |        |      |        |

0,0 -------- x    x --------          |        |      |        |

|        |      |        |

|        |      |        |

|        |      |        |

x --------    0,0 -------- x

RR x_opposite_corner_delta , y_opposite_corner_delta

Shade  rectangle  relative -  RR fills  a rectangle  with  corners

defined by  the current position and the  command parameters.  The

current  pen  position  and  up/down  state  are  unchanged.   The

rectangle edge is not drawn.

SA

Select alternate  character  set  -  SA  specifies  the  alternate

character set as current.  The alternate character set  is  chosen

with CA.

SC [xmin , xmax , ymin , ymax]  or  SC xmin,xrto,ymin,yrto,2

Scaling - SC sets user units. The units are defined by user values

for  P1 (xmin,ymin)  and P2 (xmax,ymax)  or P1 and  the  ratio  of

plotter to user units.  Using no parameters turns scaling off, and

subsequent drawing is done in plotter units (1/1016 inch).

p-9

SI [width , height]

Character size absolute - SI sets the capital letter box width and

height in centimeters.  Letter  spacing  is 1.5 *  width  and line

spacing  is 2 * height.  Using no parameters with B or A3 paper is

the same as  SI.285,.375  and  any  other  paper is  the  same  as

SI.187,.269.

SL [tan_angle]

Character slant - SL sets  the character  slant.  The parameter is

interpreted as the tangent  of  the angle from vertical.  Using no

parameter sets the slant to 0 degrees.

SM[c]

Symbol mode  -  SM sets symbol mode, which centers the symbol mode

character  at  any plotted point.  Any character from ASCII 033 to

127 except ";" sets symbol mode.  Any other character turns symbol

mode off.

SP pen_number

Select pen - SP selects a pen.  Using 0 or no parameter stores the

pen  without selecting a new pen.  This is usually done at the end

of a plot. PrintGL ignores any drawing with pen 0.

SR [width , height]

Character size relative - SR sets the capital letter box width and

height as a percentage of  P2X-P1X and P2Y-P1Y.  Letter spacing is

1.5 * width and line spacing is 2 * height. Using no parameters is

the same as SR.75,1.5.

SS

Select  standard  character  set  -  SS   specifies  the  standard

character  set as  current.  The standard character set  is chosen

with CS.

TL [positive_length [, negative length]]

Tick length -  TL  sets the  tick lengths used by XT and  YT.  The

parameters are lengths above  and  below the current position as a

percentage of P2Y-P1Y for x-axis ticks and a percentage of P2X-P1X

for y-axis ticks.

UC [[pen_control ,] x_delta , y_delta] ...

User character - UC specifies a  user defined character.  The  pen

control number is 99 for pen down or -99 for pen up. Pen movements

are relative, with x_delta in units of 1/4 the character width and

y_delta in units of 1/8 the character height.

VA

Adaptive velocity - IGNORED

p-10

VN

Normal velocity - IGNORED

VS

Pen velocity - IGNORED

WG radius , start_angle , sweep_angle [, chord_angle]

Shade  wedge - WG fills a  wedge centered at the current position.

The current  pen  position and up/down  state are  unchanged.  The

wedge edge is not drawn.

XT

X-axis tick - XT draws a  vertical line, with  length specified by

the  TL  command, at the current  position.  The  pen position and

up/down state are unchanged.

YT

Y-axis tick - YT draws a horizontal line, with length specified by

the TL  command, at  the  current position.  The  pen position and

up/down state are unchanged.

esc.(

esc.Y

Plotter on  - Esc.( and esc.Y turn the plotter  on.  This is  only

relevant with the /YY option.

esc.)

esc.Z

Plotter off - Esc.)  and esc.Z turn the plotter off. They are only

effective with the /YY option. The plotter on and off commands let

you  include comments or data to pass through to another device in

the plotfile.  These are flagged as unsupported commands if /YD is

set, as an indication that you should set /YY.

esc.@, esc.H, esc.I, esc.M, esc.N, esc.R

RS-232 device control commands - IGNORED

p-11

HP-GL/2 Commands -----------------------------------------------------

PrintGL's HP-GL/2 commands include a subset of HP-GL/2 that lets you

take  advantage  of  HP-GL/2's  pen  control  and  data compression.

PrintGL's HP-GL/2 interpreter reads all of the HP-GL commands above,

except  as noted below.  PrintGL acts like  an  HP-GL/2 plotter, not

printer,  so  form feeds are done with the PG command  not the  form

feed character.

The  LA command  is not supported  - all  lines are drawn with round

ends and round joints. And all clipping is done to the line center.

When the  output format is  HP-GL  all of the  HP-GL/2 extended  pen

control commands are  ignored.  The PT command  is needed to  ensure

solid fills look good.

AC [x , y]

Anchor corner - AC  specifies the origin of  stroked fills.  It is

handled even with HP-GL/2 off.

BP

Begin plot - BP is interpreted as PG;IN;.  It is handled even with

HP-GL/2 off.

CO "comment"

Comment - This is handled even with HP-GL/2 off.

CR [black_ref_red , green , blue , white_ref_red , green , blue]

Color range - Sets the color ranges used by the PC command.

DTt[, mode]

Define  label  terminator -  The  HP-GL/2 mode  parameter (0 print

terminator, 1 do  not print terminator) is handled even if HP-GL/2

is turned off.

FT [type [, spacing or shading [, angle]]]

Fill type - When reading HP-GL/2, PrintGL accepts type 10 (shading

percentage is the additional parameter) and 11 (raster fill number

is the additional parameter).  Type 11 fills are converted to type

10 with the shade level taken from the RF command.

For solid  and  shaded fill types, to guarantee  good results, you

must use the PW command to set the pen width  and  this width must

magnify to no  more than 24 dots in the final image.  If either of

these rules is broken, the fill may be striped rather than solid.

IR [p1x , p1y [, p2x , p2y]]

Set  scaling points relative - This is handled even if  HP-GL/2 is

turned off.

p-12

LT [pattern_number [, pattern_length, [mode]]]

Line  type -  HP-GL/2 allows the  additional  parameter to  select

relative (default)  or absolute  (in mm) pattern lengths.  This is

handled even with HP-GL/2 off.

NP [number]

Number  of  pens  -  Sets  the number of pens.  This is ignored by

PrintGL, which can handle up to 256 pens.

PC [pen [, red , green , blue]]

Pen color - Sets the pen color.

PE

Polyline encoded  - This  starts PE mode.  It is handled even with

HP-GL/2 off.

PT [thickness]

Pen  thickness  -  The  command is  not  part  of  HP-GL/2 and  is

generally ignored.  If the  output format is HP-GL  or if  you are

using /Y1 (HP-GL/2 without pen  control) then PT is interpreted as

it is in HP-GL.

PW [width [, pen]]

Pen width - Sets the  pen width.  After magnification, the maximum

width is 24 dots. PW is ignored if the output format is HP-GL.

QL

Quality level - IGNORED for HP-GL or HP-GL/2

RF [index [,width, height, pen [.. pen]]

Raster fill - RF is not fully implemented.  Instead it is read and

the  pattern is  converted to  a shade level.  Pen numbers and the

specific  pattern are ignored.  The shade level  is saved for  use

with fill type 11.

RO [angle]

Rotate coordinate system  -  When  reading HP-GL/2,  this  command

accepts 0, 90, 180, and 270 and always rotates counter-clockwise.

SV [type [, option1 [, option2]]]

Screened vectors - Sets the shade level used  to draw lines.  Only

modes 0 and 1 are supported.

TR [mode]

Transparency mode - Sets opaque or translucent mode.

p-13

UL [index [,gap1, .., gapn]]

User  line type - UL is not fully implemented.  Instead it is read

and the pattern is converted  to one  of PrintGL's eight  standard

line types. The gaps must be less that 256.

WU [type]

Width units - Sets the units used by the PW command.

esc% .. A

Enter PCL or RTL mode -  This command is treated  like esc.).  All

commands are ignored until the next esc% .. B.

esc% .. B

Enter  HP-GL/2 mode - This command  is treated like esc.(.  It  is

required before any commands are interpreted.

ADI Commands ---------------------------------------------------------

With the /YA option PrintGL can read Autodesk ADI 4.0 vector plotter

binary commands intermixed in the HP-GL. 1016 dpi coordinates or the

current HP-GL user coordinates are assumed. The table shows each ADI

command  and  the  equivalent  HP-GL  command   (as  interpreted  by

PrintGL).  In  the ADI code column, the  code  number  and  "b"s are

bytes,  and  "x" and  "y"  are words.  Eight line  types (other than

solid) are supported.

ADI command     ADI code     equivalent HP-GL

begin plot      1 b          ignored

end plot        2            PG;

move            3 x y        PUx,y;

draw            4 x y        PDx,y;

pen             5 b          SPb;

speed           6 b          ignored

line type       7 b          if b=0 then LT; else LTb,2;

raise pen       8            ignored

abort           9            ignored

p-14

HP-GL Character Sets -------------------------------------------------

PrintGL  supports all of the character sets that  are included  with

the HP 7475 and 7440/17440 except Katakana, and adds character set 5

and  the  proportional  spaced  fonts   from  the  7550.   PrintGL's

characters are similar to but not identical to HP's.

0  ANSI ASCII                           30  ISO Swedish

1  9825 character set                   31  ISO Swedish For Names

2  French/German                        32  ISO Norway Version 1

3  Scandinavian                         33  ISO German

4  Spanish/Latin American               34  ISO French

5  Special Symbols (7550 extension)     35  ISO United Kingdom

6  JIS ASCII                            36  ISO Italian

7  Roman Extensions                     37  ISO Spanish

8  Katakana - NOT SUPPORTED             38  ISO Portuguese

9  ISO IRV                              39  ISO Norway Version 2

The character sets specify a font for codes 033..127. Codes 000..032

and 128..255 are independent of the character set and are all no ops

except these.

003  default label terminator

008  backspace                 013  carriage return

009  half backspace            014  select alternate character set

010  line feed                 015  select standard character set

011  reverse line feed         032  space

Each of  the  character sets  is available as a proportional font by

adding 10 to the set number.  The proportional  fonts  are kerned to

improve  appearance (the character spacing is adjusted to compensate

for  character shape).  Use the LO  command to simplify working with

proportional fonts. To improve the appearance of large text, use the

ES command to reduce the character spacing.

Codes 65 to 81 in character set 5 are symbols designed to be used in

symbol mode (SM command) or as one character labels.  When used in a

label (LB command) these characters  are centered at the current pen

position and leave the pen position unchanged.

continued

p-15

The  HP-GL  characters  cannot  be  printed  with the standard ASCII

character set.  The file PRINTGL2.CHR is  an HP-GL file that  can be

used  to print  a  replacement for this page, showing  the character

sets. Set up PrintGL with "magnification" "magnify: 1.0", "position"

"plot:  0.0,0.0  at print origin", and "window/margins" "default" to

print this plotfile.

p-16

Program Interface ----------------------------------------------------

The  following  information lets  you  integrate PrintGL  into  your

program to print or display plotfiles.  This information is provided

for experienced programmers only - no user help will be provided.

You may not distribute PrintGL as part of another product or  on the

same disk with any commercial software (see License). Contact Ravitz

Software Inc. to inquire about special distribution licenses.

To extend  the PrintGL  setup string, set  up  multiple strings that

include only complete  options.  Include  in each string  except the

last the  option /&next_segment, next_offset  to point  to  the next

string. This links the strings together so that PrintGL can read all

of the strings.

PrintGL  can  use  your  keyboard/mouse  input  routine  instead  of

accessing BIOS or Windows  directly.  To use  this feature  add  the

segment  and offset of your keyboard routine to the /X  option.  The

routine  must be a far routine that receives one  word  parameter on

the  stack  and pops the stack on  return  (Pascal convention).  The

parameter will be:

0 -   routine returns 1 if a key is ready or 0 if no key is ready

1 -   routine waits for a key and returns it

100 - PrintGL has returned to text mode

101 - PrintGL has entered APA screen mode

102 - PrintGL has opened a destination file

110 - PrintGL return code is 0 (success)

111 - PrintGL return code is 1 (failed)

112 - PrintGL return code is 2 (user break)

Return  values should  be  put in  ax.  Keystroke  values  should be

identical to those returned by the keyboard BIOS.

100  and 101 let your program know what is on the  screen.  102 lets

you can count  how many  destination files are  created.  No  return

value is expected.

PrintGL can also use your screen output routine instead of accessing

DOS or WIndows.  To use this feature  add the  segment and offset of

your  output routine  to the /X  option.  If you are using PrintGL's

keyboard routine, precede it with "0,0,".  The routine must be a far

routine that receives one pointer on the stack and pops the stack on

return.  The pointer will point to a length delimited string (Pascal

string).  No  return information  is  expected.  This  routine  only

receives PrintGL's text data, not it graphics display data.

You will  need to  switch  to  your  data  segment to  access global

variables  from these routines.  If you add your data segment as the

fifth number in the /X option then PrintGL will set  this before  it

does a callback.  If  you want to use this feature  but not your own

screen output routine, insert "0,0," instead of the routine address.

continued

p-17

PrintGL  can read  plotfiles files or  call your program  to receive

data.  To  set  up  a  callback  routine  to transfer  data,  add /G

segment,offset  to the parameter list to specify the  address of the

routine.

This routine should, in general, switch to your  data segment,  your

stack if you need more than 1K, and your program prefix  segment and

dta if you are using DOS file access  in  the  middle of a callback.

You  can have PrintGL automatically switch  to your  data segment by

adding it as the third numeric parameter to the  /G option.  In this

case, wherever ds would set be  as documented  below, es is  instead

set.

All parameters are transferred via  register.  The callback  routine

will be called with these parameters:

open file

ax = $3d00

cx = 0

dx = offset of filename

ds = segment of filename (filename is null delimited string)

returns handle in ax, sets carry flag on error

Since you  set the filename in the PrintGL parameter list, it is

not important. The returned handle is also not important.

close file

ax = $3e00

bx = handle

read data

ax = $3f00

bx = handle

cx = amount of data desired

dx = offset of buffer to load data

ds = segment of buffer to load data

returns amount of data loaded in ax (must be <= cx and >= 1),

sets carry flag on error

You can return as little as 1 byte of data or as much as cx.

Setting  the  program  prefix  segment  is  accomplished  via  these

undocumented DOS calls (int $21)

ax = $5100 returns psp in bx

ax = $5000 and bx = psp sets the psp.