SetIt

SetIt is a DOS setup utility for the HP100/200LX Palmtops.

Copy SetIt.exe to any directory in your path, than type SetIt on the command

line to use it.

SetIt will display the command line options if no arguments are given or if the

'H' option is given.

SetIt process the command line arguments in the order given so BA BN leaves the

battery setting at Nicad,  by the same token M5 S5 leaves the timeout set to 5

seconds NOT 5 minutes and 5 seconds.

SetIt is not case sensitive so BN bn Bn bN all mean the same thing.

The 'D' option displays the machines current setup.  To have the current setup

written to a file instead of the screen add a file name after the D as in

setit dsetit.txt

When SetIt is displaying the machine status it switches the screen to CGA 80 x25

after the user presses a key to continue SetIt switches the display back to what

it was when SetIt started.

When SetIt displays the help screen it switches the display to CGA 80 x 25 mode

and leaves the user at the dos prompt making it easy to enter the command you

want

SetIt is ideal for use in batch files.  Setit returns error codes for using in

batch files,  a negative return code indicates an error a zero return code

indicates no error SetIt will never return a positive return code

SetIt is FREE.

Although I have had no problems with SetIt I TAKE NO RESPONSIBILITY FOR WHAT

SETIT MIGHT OR MIGHT NOT DO TO YOUR MACHINE.