BACKLITE V2.0

(c)2002 Stefan Peichl, Heidelberg, Germany

http://hplx.pgdn.de

eMail: hplx@pgdn.de

BACKLITE is a driver for Daniel Hertrich's backlight project.

For further information about the HP palmtop backlight and how

to install it, please visit:

http://www.daniel-hertrich.de/backlight/

REVISION HISTORY

Version 2.0  -Countdown timer added

(20-DEC-02)  -Non resident mode added

-Screen inversion removed

-Hotkey changed to Fn-X

Version 1.0  -First public release

(04-NOV-99)

HOW TO USE BACKLITE

The main purpose of BACKLITE is to enable you to control the

backlight of your palmtop by software. This is done through

pin 48 of the CPU, where the backlight receives its power from.

Just call BACKLITE from the command line without any switches

to see the Help/Status screen. It informs you about the actual

status of the backlight.

Add the line BACKLITE/L to your AUTOEXEC.BAT in order to load

the driver resident. Then use the hotkey Fn-X to toggle the

backlight on and off.

BACKLITE may be used in resident, non-resident and mixed mode.

The switches /F and /N work in resident and non-resident mode

and are useful to control the backlight from batch files or the

command line.

All other switches only affect the resident loaded driver.

COMMAND LINE SWITCHES

Resident and non-resident mode:

/F   = Switch backlight off

/N   = Switch backlight on

Resident mode only:

/L   = Load Resident

/U   = Unload Resident

/S   = Sound Toggle

/0-9 = Timeout in minutes  (0 = no timeout set)

Examples:

BACKLITE/L3 :Load resident with a timeout of 3 minutes

BACKLITE/0  :Disable the timeout for the resident loaded driver

BACKLITE/S  :Disable the hotkey sound

BACKLITE/F  :Switch off the light

DISCLAIMER

This software is copyrighted freeware.

The author disclaims all warranties, expressed or implied.

The author assumes no liability for damages, direct or

consequential, which may result from the use of this software.