------------------------------ setup.doc -------------------------------

Wintab Digitizer Services and Drivers -- installation instructions.

Copyright  LCS/Telegraphics, 1991-1996.  All Rights Reserved.

--------------------------------------------------------------------------

WINDOWS 3.1 INSTALLATION

--------------------------------------------------------------------------

* Prepare the Device

Make sure your device is properly connected and ready for use according to

manufacturer's instructions.

* Install the Device Driver

1. Install tablet as a system pointing device:

To use a tablet as your only pointing device (system pointing device),

you must first install AYEMOUSE.DRV to keep Windows pointing enabled.

This driver is provided with your setup kit.

Use Windows' SETUP.EXE, either from the DOS prompt in your Windows directory,

or from within Windows.

Follow the directions to change your mouse driver, and choose 'Other'

from the end of the list of mouse drivers.  When asked for a disk or

pathname, enter the path of your Wintab driver disk or directory.

Now install the Wintab Driver as described below.

To install Wintab Driver run the setup program of your Wintab driver disk.

Choose KYE MM Tablet. The driver will be installed  automaticly.

2. Install tablet as a pointing device:

If you install your tablet coexsisting with a mouse it becomes a pointing

device.

To install Wintab Driver run the setup program of your Wintab driver disk.

Choose KYE MM Tablet. The driver will be installed  automaticly.

* If you have "Windows for Pen Computing"

Use the Drivers Applet in the Windows Control Panel to configure the Wintab

Pen Interface, WTPEN.DRV.  This pen driver makes any Wintab-compatible

pointing device pen-compatible.

* Wintab Drivers and Upgrading to Windows 95

If you have installed a Wintab driver and you wish to upgrade your

configuration to Windows 95, you must remove the Wintab drivers before

upgrading and re-install them once the upgrade is complete.

--------------------------------------------------------------------------

INI-file STRINGS USED

--------------------------------------------------------------------------

* AYEMOUSE.DRV

Installing the AYEMOUSE.DRV driver changes the following strings

in SYSTEM.INI:

[boot]

mouse.drv=ayemouse.drv

[boot.description]

mouse.drv=Enable Windows pointing without a mouse.

* Installable Drivers

Installable drivers use an "alias" in performing their SYSTEM.INI

modifications.  The alias is usually the same as the driver file name,

minus the extension.  SETUP.EXE will add the alias to the following line:

[boot]

drivers=mmsystem.dll <other aliases may appear here> <alias>

SETUP.EXE will then add the following line:

[drivers]

<alias>=<driver file name>

* 386 Enhanced Mode Virtual Device Drivers:

If you are using the installable driver, find the line in the [drivers]

section of SYSTEM.INI that contains the filename of the driver you are

using.  The portion to the left of the equals sign is the driver's "alias".

The Wintab driver will add the following section to SYSTEM.INI:

[vsertd.386]

<Alias>=<X>

where Alias is the driver's alias discovered from the [drivers] section,

and X is the com port number, as above.

* Wintab Control Panel and Wintab Features

The driver's setup program adds the Wintab Features DLL to the load=

line of WIN.INI's [windows] section, as follows:

[windows]

load= <other programs, if any> WTKYEF.EXE

It also adds a line t/o CONTROL.INI's section [MMCPL] as follows:

[MMCPL]

WTKYEC=wtkyec.exe

--------------------------------------------------------------------------

REMOVING THE DRIVER FROM WINDOWS 3.1

--------------------------------------------------------------------------

Merely using the Drivers applet in the Windows Control Panel does not

remove all parts of the Wintab system.  To remove the tablet driver

from your system, follow these steps (only some of these would be

done by the Drivers applet.)  See also the previous section for more

detail on the INI files.

1) If you installed AYEMOUSE.DRV, replace it now with an actual

mouse driver.

2) Edit SYSTEM.INI:  In section [boot], remove the tablet's

alias from the "drivers=" line.  In the [drivers] section,

remove the entire line beginning with the tablet alias.

In the section [386Enh] remove the line reading:

"driver=vsertd.386"  Look for a section headed [vsertd.386].

If this section exists, remove it.

3) Edit WIN.INI.  Look under [windows] for the line starting with

"load=".  Remove the program name WTKYEF.EXE from this line.

4) Edit CONTROL.INI.  In the section [MMCPL] delete the line

"WTKYEC=WTKYEC.EXE".

5) Restart Windows.  Optionally, delete some or all of these files

from your Windows and System directory:

Windows dir:

WTKYEC.EXE

WTKYEC.HLP

WTKYEF.EXE

WINTAB.INI

System dir:

AYEMOUSE.DRV

VSERTD.386

WINTAB.DLL

WINTAB32.DLL

WTMMKYE.DRV

WTPEN.DRV

WTT16.DLL

--------------------------------------------------------------------------

ADVANCED INSTALLATION OPTIONS

--------------------------------------------------------------------------

* Nonstandard Com Ports

If you have a non-standard com port IRQ line or IO port base address, add the

following lines to the [386Enh] section of SYSTEM.INI:

COM<X>IRQ=<I>

COM<X>BASE=<B>

where I is an IRQ line number from 2 through 15, B is a base address in

hexadecimal, and X is the com port number.  For example:

COM3IRQ=5

COM3BASE=338

--------------------------------------------------------------------------

FILE DESCRIPTIONS

--------------------------------------------------------------------------

The files in this release are as follows:

AYEMOUSE.DRV        Null mouse driver (for users with no mouse).

SETUP.DOC           Installation instructions (this file).

SETUP.EXE           Installation program for Windows 3.1.

VSERTD.386          VxD, Virtual Device driver.

WINTAB.DLL          16-bit Wintab API & functionality.

WINTAB32.DLL        32-bit Wintab API & functionality.

WTKYEC.EXE          Wintab Control Panel DLL.

WTKYEC.HLP          Helpfile for Wintab Control Panel.

WTKYEF.EXE          Wintab Features DLL.

WTMMKYE.DRV         Device-specific driver, MM-Format tablets.

WTPEN.DRV           Wintab Pen Driver (for Pen Windows).

WTT16.DLL           16-bit interface for dual use under Win32.

--------------------------------------------------------------------------

Copyright  LCS/Telegraphics, 1991-1996.  All Rights Reserved.